CPE301 – SPRING 2019

# Design Assignment 2B

Student Name:  Meral Abu-Jaser

Student #: 5003137888

Student Email: abujaser@unlv.nevada.edu

Primary Github address: https://github.com/MeralAbuJaser/Submission_da.git

Directory: https://github.com/MeralAbuJaser/Submission_da/tree/master/DB2B/DA2B

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7.0

-debugger

-simulator

-assembler

Atmega 328pb  Sheild           logic analizer

male to male jumper

using pins PC3, PB3, PB2

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

C code

#define F_CPU 16000000L

#include <avr/interrupt.h>

#include <avr/io.h>

#include <util/delay.h>

int main(void){

DDRB |= (1<<3);		//DDRB.3 is output

PORTB &=~(1<<3);	//PORTB.3 is output on low

EICRA = 0X02;		//external interrupt on falling edge

EIMSK = (1<<INT0); 	//external interrupt INT0

sei ();

while(1){

PORTB |=(1<<3); //"LED ON"

_delay_ms(412); //delay 412ms

PORTB &=~(1<<3);//"LED OFF"

_delay_ms(337); //delay 337ms

}

return 1;

}

ISR (INT0_vect){

DDRB &= ~(1<<3);		//PORTB.3 output low DDRB.3

DDRB |= (1<<2);			//PORT.2 output high

_delay_ms(2000);		//delay 2s

PORTB &=~(1<<2);		//PORTB.3 output High

DDRB &=~(1<<2);			//PORTB.2 output LOW DDRB.2

DDRB |= (1<<3);			//PORT.3 output high

}

Assembly code

.ORG 0

JMP LOOP

.ORG 0x02

JMP EX0_ISR			//initializing stack

LDI R22,HIGH(RAMEND)		//

OUT SPH,R22			//

LDI R22,LOW(RAMEND)		//

SBI DDRB, 2			//portb is set as output

CBI PORTB, 3		//portb.3 is set to low

LOOP:

LDI R22, 0X2		//falling edge triggered

STS EICRA, R22		//INT0 controller

SBI DDRB, 3

SBI PORTB, 3		//set to high

CALL DELAY2			//337.5ms delay

CBI PORTB, 3		//set to low

CALL DELAY1			//412.5ms delay

LDI R22, 1 << INT0	//falling edge triggered

OUT EIMSK, R22

SEI			//enabling interrupts

JMP LOOP

EX0_ISR:

CBI DDRB, 3

SBI DDRB, 2

CALL DELAY3 //delaying by 2s

CBI DDRB, 2 //outputting 0 to PB2

DELAY1://412.5ms delay loop

LDI R16, 230

LDI R17, 243

LDI R18, 32

AGAIN1:

DEC  R16

BRNE AGAIN1

DEC  R17

BRNE AGAIN1

DEC  R18

BRNE AGAIN1

RET

DELAY2://337.5ms delay loop

LDI R16, 250

LDI R17, 237

LDI R18, 37

AGAIN2:

DEC  R16

BRNE AGAIN2

DEC  R17

BRNE AGAIN2

DEC  R18

BRNE AGAIN2

RET

DELAY3://2 second deley loop

LDI R16, 164

LDI R17, 255

LDI R18, 255

AGAIN3:

DEC  R16

BRNE AGAIN3

DEC  R17

BRNE AGAIN3

DEC  R18

BRNE AGAIN3

RET

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=phgdcTggWX8&t=0s

- GITHUB LINK OF THIS DA

https://github.com/MeralAbuJaser/Submission_da/tree/master/DB2B/DA2B

“This assignment submission is my own, original work”.